### 1. Which ARM instruction is used for unconditional branching?

### a) B
b) BL
c) BX
d) CMP

Answer: a) B
Explanation: The B instruction is used for an unconditional branch to another address. It simply transfers control to the target address without any condition.2. Which keyword in C is used to define a variable with a fixed memory location?

a) volatile
b) const
c) register
d) static

Answer: d) static
Explanation: The static keyword ensures that a variable retains its value across function calls and has a fixed memory location in the data segment.3. What does the PUSH instruction do in ARM assembly?

a) Moves a value to a register
b) Writes a value to memory
c) Stores registers onto the stack
d) Loads registers from memory

Answer: c) Stores registers onto the stack
Explanation: The PUSH instruction saves the contents of one or more registers onto the stack, decreasing the stack pointer.4. How does the CMP instruction work in ARM assembly?

a) Compares two values and stores the result
b) Compares two values and updates condition flags
c) Adds two values
d) Moves a value to a register

Answer: b) Compares two values and updates condition flags
Explanation: The CMP (Compare) instruction subtracts one value from another and updates the condition flags based on the result, without storing it.5. Which section in memory contains uninitialized global variables?

a) Code section
b) Data section
c) BSS section
d) Stack section

Answer: c) BSS section
Explanation: The BSS (Block Started by Symbol) section is used for uninitialized global and static variables, which are initialized to zero at runtime.6. What does BL stand for in ARM assembly?

a) Branch and Link
b) Branch Loop
c) Block Load
d) Block Link

Answer: a) Branch and Link
Explanation: The BL (Branch and Link) instruction branches to a subroutine and saves the return address in the lr (Link Register).7. What is the result of executing AND r0, r1, r2 in ARM assembly?

a) Logical OR between r1 and r2, stored in r0
b) Logical AND between r1 and r2, stored in r0
c) Logical XOR between r1 and r2, stored in r0
d) Logical NOT of r1, stored in r0

Answer: b) Logical AND between r1 and r2, stored in r0
Explanation: The AND instruction performs a bitwise AND operation between the contents of r1 and r2 and stores the result in r0.8. What is the purpose of the volatile keyword in C?

a) Improves performance
b) Prevents the compiler from optimizing the variable
c) Makes the variable global
d) Makes the variable read-only

Answer: b) Prevents the compiler from optimizing the variable
Explanation: The volatile keyword informs the compiler that the value of the variable can change at any time, preventing the compiler from applying optimizations.9. What happens when a function is called in ARM assembly?

a) The return address is stored in the lr register
b) The return address is stored in the sp register
c) Registers are automatically saved to the stack
d) The function executes without saving any address

Answer: a) The return address is stored in the lr register
Explanation: When a function is called, the address of the next instruction is saved in the lr (Link Register), so the program can return after the function completes.10. Which ARM instruction is used to perform a right logical shift?

a) LSL
b) LSR
c) ASR
d) ROR

Answer: b) LSR
Explanation: The LSR (Logical Shift Right) instruction shifts the bits of a register to the right, inserting zeros into the high-order bits.11. What is the default size of a char in ARM Cortex-M?

a) 8 bits
b) 16 bits
c) 32 bits
d) 64 bits

Answer: a) 8 bits
Explanation: On ARM Cortex-M, the char data type is typically 8 bits (1 byte) as per the standard C specifications.12. What does LDM stand for in ARM assembly?

a) Load Multiple Registers
b) Load Memory
c) Logical Data Move
d) Load and Move

Answer: a) Load Multiple Registers
Explanation: The LDM (Load Multiple) instruction loads data from memory into multiple registers at once.13. Which directive is used to define a constant in ARM assembly?

a) .data
b) .const
c) .equ
d) .set

Answer: c) .equ
Explanation: The .equ directive is used to define a constant value in ARM assembly.14. How many general-purpose registers are available in ARM Cortex-M?

a) 8
b) 13
c) 16
d) 32

Answer: b) 13
Explanation: ARM Cortex-M processors have 13 general-purpose registers (r0 to r12), along with other special-purpose registers.15. What is the purpose of the NOP instruction?

a) Execute a system call
b) Perform no operation
c) Stop execution
d) Reset the processor

Answer: b) Perform no operation
Explanation: The NOP (No Operation) instruction does nothing and is often used for timing adjustments or alignment.16. How is a conditional branch performed in ARM assembly?

a) Using B
b) Using CMP
c) Using B with condition codes
d) Using MOV

Answer: c) Using B with condition codes
Explanation: Conditional branches in ARM use the B instruction with condition codes (e.g., BEQ, BNE, etc.) to branch based on specific conditions.17. Which register holds the stack pointer in ARM?

a) lr
b) sp
c) pc
d) r0

Answer: b) sp
Explanation: The sp (Stack Pointer) register points to the top of the stack in ARM processors.18. Which keyword in C is used for a variable that must not be changed?

a) volatile
b) static
c) const
d) extern

Answer: c) const
Explanation: The const keyword makes a variable read-only, meaning its value cannot be changed after initialization.19. What does TST do in ARM assembly?

a) Tests the sign of a value
b) Performs a bitwise AND and updates flags
c) Shifts bits to the left
d) Loads a value into a register

Answer: b) Performs a bitwise AND and updates flags
Explanation: The TST (Test) instruction performs a bitwise AND operation between two values and updates the condition flags without storing the result.20. Which ARM instruction clears a register to zero?

a) MOV r0, #0
b) CLR r0
c) SUB r0, r0, r0
d) Both a and c

Answer: d) Both a and c
Explanation: Both MOV r0, #0 and SUB r0, r0, r0 can be used to clear a register by setting it to zero. The former is more direct, while the latter uses subtraction.