7. (a) : Finding the Maximum of 20 Integer Numbers and Storing it in Register (r8)

.global _start

_start:

LDR     r0, =numbers          @ Load address of the array

LDR     r1, [r0], #4               @ Load the first number and increment address

MOV     r8, r1                      @ Initialize r8 with the first number (max)

MOV     r10, #19                 @ Counter for the remaining 19 numbers

find_max_loop:

LDR     r1, [r0], #4                @ Load the next number

CMP     r1, r8                        @ Compare the number with the current max in r8

BLE     skip_max                   @ If r1 <= r8, skip updating max

MOV     r8, r1                        @ Update max if r1 > r8

skip_max:

SUBS    r10, r10, #1               @ Decrement the counter

BNE     find_max_loop         @ Repeat the loop if counter not zero

B       end                                 @ Jump to the end of the program

numbers:                                     @ Array of 20 numbers

.word  25, 14, 98, 33, 67, 89, 22, 43, 56, 87

.word  12, 65, 11, 90, 34, 71, 82, 93, 23, 58

end:

B       end                                    @ Infinite loop to stop execution

7(b): Finding the Minimum of 20 Integer Numbers and Storing it in Register (r9)

.global _start

_start:

LDR     r0, =numbers          @ Load address of the array

LDR     r1, [r0], #4               @ Load the first number and increment address

MOV     r9, r1                      @ Initialize r9 with the first number (min)

MOV     r10, #19                 @ Counter for the remaining 19 numbers

find_min_loop:

LDR     r1, [r0], #4                @ Load the next number

CMP     r1, r9                        @ Compare the number with the current min in r9

BGE     skip_min                  @ If r1 >= r9, skip updating min

MOV     r9, r1                       @ Update min if r1 < r9

skip_min:

SUBS    r10, r10, #1              @ Decrement the counter

BNE     find_min_loop         @ Repeat the loop if counter not zero

B       end                                  @ Jump to the end of the program

numbers:                                     @ Array of 20 numbers

.word  25, 14, 98, 33, 67, 89, 22, 43, 56, 87

.word  12, 65, 11, 90, 34, 71, 82, 93, 23, 58

end:

B       end                                    @ Infinite loop to stop execution

3(a) Triangle Validity Check (Print 0 in Seven-Segment Display for Invalid Sides)

.global _start

.equ led, 0xff200020

_start:

ldr r4, =led

MOV r6, #5

MOV r7, #3

MOV r8, #9

Con1:

ADD r3, r6, r7

CMP r3, r8

BGT Con2                                   @ If r3 (8) is greater than r8 (9), branch to Con2

BLE False                                    @ If not greater, branch to the False label (conditions not satisfied)

Con2:

ADD r3, r7, r8

CMP r3, r6

BGT Con3                                       @ If r3 (12) is greater than r6 (5), branch to Con3

BLE False                                         @ If not greater, branch to the False label

Con3:

ADD r3, r6, r8

CMP r3, r7

BGT True                                             @ If r3 (14) is greater than r7 (3), branch to True

BLE False

True:

MOV r5, #0b0000110

STR r5, [r4]

B exit                                        @ Branch to exit

False:

LDR r5, =#0b00111111

STR r5, [r4]

B exit

exit:

MOV r7, #1                     @ exit by setting r7 to 1

SWI 0

3(b) Rectangle Area and Perimeter Calculation (Print Result in Seven-Segment Display)

.equ sevensegment, 0xff200020

.global _start

_start:

LDR r4, =sevensegment

MOV r1, #4                  // Length

MOV r2, #13                 // Width

MUL r3, r1, r2                                       // Calculating Area , r3 = Length * Width

LDR r5, =#0b000011001101101      // Area representation

STR r5, [r4]                                         // Store area on the 7-segment display

// Perimeter

ADD r6, r1, r2                                                         // r6 = Length + Width

ADD r6, r6, r6                                                       // r6 = 2 * (Length + Width)

LDR r7, =#0b100111101100110                      // Perimeter representation

STR r7, [r4]                                                          // Store perimeter on the 7-segment display

SWI 0                        // Exit